This folder contains selected historic airphotos for mapping tidewater glacier fronts

First goal is to make a complete set of fronts for the period 1960-1970.

This is half-done in the shapefile: fronts1960_extended.shp

Overview of remaining fronts to be digitized from airphotos: remaining_fronts_1960s.shp

Reference data for coastline and historic glacier fronts in folder: coast&fronts

Additional background GIS data from WMS services here: geodata.npolar.no

(useful data can be: basiskart, ortofoto, satellitt-mosaikk)

Workflow:

- Go through the list of remaining fronts and available aiphotos

- Find the airphoto in the image catalog, then georeference it vs. stable non-glacier parts of the coastline, preferably on both sides of the front to be mapped. Islands are also good references and can help to improve the contro-point distribution. In some cases, it could help to mosaic adjacent scenes together before georeferencing them in one go. Or a single photo can be georeferenced separately twice to optimize the fit in each area.

- Digitize all fronts in the georeferenced airphoto into the fronts1960 shapefile. Each line should be continuous through the reference polygons and a little bit beyond (no snapping). Check that stable parts of the coastline fit well on either side of each glacier (but keep in mind that moraine- and sediment-type coastlines can have changed too). Note the image name (and date if given) in the attribute table.

- Work through all remaining fronts, year-by-year or region-by-region. If issues with the suggested available image, then check the comment-attribute for alternatives. Make notes in attribute table if fronts/images are missing, incomplete or of poor quality.

- If it doesn't slow down the work -> save the georeferenced images for later reference.